MATLAB Codes for Calculating Transmittance/Reflectance/Absorbance of Asymmetrical Cavity with Sb₂Se₃ Layer

Main Function: ให้บันทักเป็นไฟล์ที่มีนามสกุล .m

clear; clc;

% --- User Parameters ---

lambda = linspace(400e-9, 1800e-9, 200);     % Wavelength [m]

theta_in = linspace(0, 90, 91);               % Incident angles [deg]

n_air = 1;                                    % Incident medium

n_sub = 1.45;                                 % Substrate

n_DBRH = 2.0; n_DBRL = 1.45;                  % DBR indices Si3N4/SiO2

d_DBRH = 100e-9; d_DBRL = 100e-9;             % DBR layer thicknesses [m]

n_Sb2Se3 = 3.3 + 0.01i;                     % Sb₂Se₃ (amorphous, example)

%n_Sb2Se3 = 4.1 + 0.25i;                     % Sb₂Se₃ (crystalline, example)

d_Sb2Se3 = 80e-9;                             % Sb₂Se₃ thickness [m]

N = 5;    % Number of layers in upper DBR (on top/Sb₂Se₃ side)

M = 7;    % Number of layers in bottom DBR (on substrate side)

% --- Build DBR Layer Stack ---

layers_n = n_air;

layers_d = 0;

% Upper DBR: alternating N layers (starts with high index)

for i = 1:N

if mod(i,2)==1, layers_n(end+1) = n_DBRH; layers_d(end+1) = d_DBRH;

else,           layers_n(end+1) = n_DBRL; layers_d(end+1) = d_DBRL;

end

end

% Sb₂Se₃ layer

layers_n(end+1) = n_Sb2Se3;

layers_d(end+1) = d_Sb2Se3;

% Bottom DBR: alternating M layers (starts with high index)

for i = 1:M

if mod(i,2)==1, layers_n(end+1) = n_DBRH; layers_d(end+1) = d_DBRH;

else,           layers_n(end+1) = n_DBRL; layers_d(end+1) = d_DBRL;

end

end

% Substrate

layers_n(end+1) = n_sub;

layers_d(end+1) = 0;

% --- Initialize Results ---

T = zeros(length(theta_in), length(lambda));

R = zeros(length(theta_in), length(lambda));

A = zeros(length(theta_in), length(lambda));

% --- Main Calculation Loop ---

for it = 1:length(theta_in)

theta0 = theta_in(it) * pi/180; % deg -> rad

for il = 1:length(lambda)

lam = lambda(il);

[t, r] = tmm_asymmultilayer(layers_n, layers_d, lam, theta0, 'TE');

th_out = theta_sub(layers_n, theta0);

% Transmission: power normalization for oblique incidence

T(it, il) = abs(t)^2 * real((layers_n(end)*cos(th_out))/(layers_n(1)*cos(theta0)));

R(it, il) = abs(r)^2;

A(it, il) = 1 - T(it, il) - R(it, il);

end

end

% --- Visualization ---

figure(1);

contourf(lambda*1e9, theta_in, T, 40, 'LineColor','none');

xlabel('Wavelength (nm)');

ylabel('Incident angle (deg)');

title(['Transmittance, N = ' num2str(N) ', M = ' num2str(M)]);

colormap('jet');

colorbar;

figure(2);

contourf(lambda*1e9, theta_in, R, 40, 'LineColor','none');

xlabel('Wavelength (nm)');

ylabel('Incident angle (deg)');

title(['Reflectance, N = ' num2str(N) ', M = ' num2str(M)]);

colormap('jet');

colorbar;

figure(3);

contourf(lambda*1e9, theta_in, A, 40, 'LineColor','none');

xlabel('Wavelength (nm)');

ylabel('Incident angle (deg)');

title(['Absorbance, N = ' num2str(N) ', M = ' num2str(M)]);

colormap('jet');

colorbar;

MATLAB Codes

Sub-Function 1: ให้บันทึกเป็นไฟล์แยกและตั้งชื่อว่า tmm_asymmultilayer.m

% --- Functions ---

function [t, r] = tmm_asymmultilayer(n, d, lambda, theta0, pol)

k0 = 2*pi/lambda; N = length(n);

theta = zeros(1,N); theta(1) = theta0;

for j = 2:N

theta(j) = asin(n(j-1)*sin(theta(j-1))/n(j));

end

M = eye(2);

for j = 2:N-1

beta = k0*n(j)*d(j)*cos(theta(j));

P = [exp(-1i*beta), 0; 0, exp(1i*beta)];

if strcmpi(pol, 'TE')

r_j = (n(j-1)*cos(theta(j-1)) - n(j)*cos(theta(j)))/(n(j-1)*cos(theta(j-1)) + n(j)*cos(theta(j)));

t_j = 2*n(j-1)*cos(theta(j-1))/(n(j-1)*cos(theta(j-1)) + n(j)*cos(theta(j)));

else

r_j = (n(j)*cos(theta(j-1)) - n(j-1)*cos(theta(j)))/(n(j)*cos(theta(j-1)) + n(j-1)*cos(theta(j)));

t_j = 2*n(j-1)*cos(theta(j-1))/(n(j)*cos(theta(j-1)) + n(j-1)*cos(theta(j)));

end

I = (1/t_j)*[1 r_j; r_j 1];

M = M * I * P;

end

if strcmpi(pol, 'TE')

rN = (n(N-1)*cos(theta(N-1)) - n(N)*cos(theta(N))) / (n(N-1)*cos(theta(N-1)) + n(N)*cos(theta(N)));

tN = 2*n(N-1)*cos(theta(N-1)) / (n(N-1)*cos(theta(N-1)) + n(N)*cos(theta(N)));

else

rN = (n(N)*cos(theta(N-1)) - n(N-1)*cos(theta(N))) / (n(N)*cos(theta(N-1)) + n(N-1)*cos(theta(N)));

tN = 2*n(N-1)*cos(theta(N-1)) / (n(N)*cos(theta(N-1)) + n(N-1)*cos(theta(N)));

end

I_N = (1/tN)*[1 rN; rN 1];

M = M * I_N;

t = 1/M(1,1); r = M(2,1)/M(1,1);

end

Sub-Function 2: ให้บันทึกเป็นไฟล์แยกและตั้งชื่อว่า th_sub.m

function th_sub = theta_sub(n, theta0)

theta = theta0;

for j = 2:length(n)

theta = asin(n(j-1)*sin(theta)/n(j));

end

th_sub = theta;

end

function th_sub = theta_sub(n, theta0)

% Calculates the transmitted angle in the final substrate layer

% n ... vector of refractive indices (incident to substrate)

% theta0 ... incident angle in radians

theta = theta0;

for j = 2:length(n)

theta = asin(n(j-1) * sin(theta) / n(j));

end

th_sub = theta;

end